Tank Abandonment Procedures

Instructions A licensed sewage disposal service must pump the tank(s) and remove all its contents. Attach a copy of the pumper's receipt to this form and submit it to the Community Development office for inspection. The inspection will usually take place within the next 2 working days, and only then can the final covering be done.

Select ONE of the following 3 options:

Removal Check with the local landfill or local recycler before transporting the old tank. Not all locations are approved to accept used septic tanks. Mark or flag the location where the tank location was in use prior to requesting an inspection.

Fill in place The top of the tank shall be removed before filling. Destroy steel or plastic tops and cave in concrete lids. Fill tanks with sand or bar-run gravel and pack the tank so as to eliminate void areas. For inspection; fill tanks to the tops of the vertical sides and request an inspection using the instructions below. After the inspection is completed, you will need to fill the tank to final grade.

Crush and bury After pumping the tank, crush the tank so that there are no void areas.  Leave the crushed tank visible for the inspection.

| Permit and Applicant Information |  | Existing Septic Permit #: |  |  |
| --- | --- | --- | --- | --- |
| Property Address: |  |  | Phone#: |  |
| Applicant Name: |  |  | Date: |  |
| Reason for Abandonment | Connecting to Sewer | Installing new tank |  | Other __________________ |

I certify that the above information is true and complete to the best of my knowledge and that the decommissioning of this septic system on the above stated lot has taken place in accordance with the above procedures and the OAR 340-71-0185 requirements.

Contractor's Signature: ________________________________________________Date: _____________________

Remember to attach a copy of the pumper's receipt with this form.

Note: Components of the Decommissioned System

Drainfields can be left intact. They should not however, be excavated for a year after the last use. Distribution boxes should be pumped out by a licensed pumper and removed or filled in location. Sand Filter mounds can be left in place, but if removed, they should not be excavated for a year after the last use.

EHS Approval:_______________________________

Date:______________________________________

Department of Planning &

Development

210 S. W. 2nd St.

541-265-4192

541-265-6945 Fax

lconsite@co.lincoln.or.us